Ice Cream Melting: Estimating Time To Drip

CHE 447, Spring 2026

Group 6

April 21, 2026

Kaden Corrow-Webb, Linden Olmstead, Zijian Wang

## Introduction

Ice cream melting is one of the most universal and unfortunate experiences shared among almost everyone and represents a significant process of heat and mass transfer in everyday life. Heat transfer in the ice cream, mass transfer into the sogginess of the cone, and fluid mechanics in the drip of the melted ice cream. These all contribute to this being a coupled transient heat-and-mass transfer problem involving a phase change and physical melting into the cone holding up the ice cream. Heat transfer is governed by transient conduction within the ice cream and convection with the outside environment at the exposed surfaces, contributing to the heat of phase change for the melting to take place. The purpose of this report is to understand how the time-dependent melting of ice cream takes place and how the relative fat content and composition of the cream affects this process. Key parameters to be estimated include:

- Time for enough ice cream to melt to form a drip

- Relative sogginess of the cone over time as a result of ice cream melting

- External convection coefficient (h)

## Assumptions

The following assumptions are made for modelling and time calculations in this report:

- Ice cream is spherical with a fixed radius, R, and it is on top of a cone, which obscures some of the volume of the sphere from free convection.

- Free convection is happening at the outside of the ice cream bulk and outside the cone

- Heat conduction through the wall of the ice cream cone is taking place

- Ice cream is undergoing diffusion through the cone, making it more soggy and reducing its structural integrity. This process carries heat with it.

- All heat into ice cream goes into temperature increase until phase change, after which ice cream will continue melting.

## Methods

The governing equation behind heat transfer is the following transient heat-conduction equation:

ρCp∂T∂t=∇⋅k∇T

The above energy equation is solved using a numerical analysis tool in COMSOL. T is the temperature profile solved in the program, with space and time dependencies in the system.

There are two conditions necessary for this solution:

Initial Condition: Uniform initial temperature in entire system

Tx,0=T0

Boundary Condition: All outer surfaces are exposed to air undergoing free convection

-n⋅(-k∇T)=hT-T∞

where h is the external heat-transfer coefficient, T∞ is the ambient air temperature, and n is the outward normal direction.

Since the temperature of the ice cream is not uniform, the built in integration tools are used to solve for total energy put into the system over time, which is related to the mass of the ice cream in the form of density by volume. The governing heat transfer equation is used for this analysis:

Q=mCpΔT

For a differential volume element of ice cream, its mass is represented by dm=ρdV. For the rise from T0 to T which is solved using COMSOL, the total heat absorbed may be found. The total heat absorbed by the ice cream over time is represented by the following:

E(t)=Vice ρiceCp,iceT−T0dV

where ρice is the density of the ice cream (which is dependent on the composition of the ice cream) and Cp,ice is the specific heat capacity of the ice cream.

Initially, all energy into the system will only go towards the heating of the ice cream until its melting temperature of Tm=273.15 K. Using the governing heat transfer equation above and the volume of the ice cream with its density, the melting onset time is defined as the point at which

Eice(t)≥Qreq, melt

Past this point, any additional absorbed energy into the system will only go towards phase change. Thus, the melted mass is defined as:

Mmelt (t)=max0,Eice (t)−Qreq, melt ΔHmelt

All excess energy past the melting point temperature will just continue to go towards further melting of ice cream on the interior, hence the division by the latent heat of phase change.

## Results

Using a constant specific heat capacity, the mass of the ice cream, and the temperature change from initial to the melting point, the total heat required to warm the entire ice cream mass is found:

Qreq , melt =mice Cp, ice Tm−T0 = [value]

Using the total energy absorbed by the system with COMSOL simulations over time, we can find the time at which the total energy has become greater than or equal to the heat required to melt it, which will become the melt onset time.

[ex: some type of figure or data showing time to reach melting point against volume of ice cream]

Any heat past this point will only go into phase change based on the latent heat of phase change. For a given volume, the melted mass of the ice cream is as follows:

[table of original mass and mass of ice cream melted after X amount of time, for example]

## Discussion and Conclusion

## References (tentative, just topical articles now)

- https://hal.science/hal-01549291/document

  - Numerical analysis of an ice cream block melting to determine the ice cream structure as it changes over time

- http://www.worldcool.co.th/images/Knowlage/13-Unfrozen-Composition-Data.PDF

  - Latent heat of fusion (melting) around 200 kJ/kg

- https://www.researchgate.net/publication/222531132_Experimental_data_and_modeling_of_thermal_properties_of_ice_creams

  - Latent heat of fusion around 200 J/g (same as above link)

  - j indicates each component, essentially the specific heat capacity is dependent on the sum of components and their respective mass fraction. Do not need to use this but its something to consider